Micropython ESP32 Camera

- Install custom firmware (lemariva/micropython-camera-driver: add camera support to MicroPython)

While installing firmware, connect GPIO0 to GND and hold reset button at the same time for several seconds then release.

- Upload the necessary program files to the device

- Press Run & Copy the Network ip to your browser

Experiment Micropython & ESP-IDF Custom Firmware

(Only works with ESP-IDF v.4.4.4)

ESP Tools for Build Firmware

Follow these steps:

- Standard Toolchain Setup for Linux and macOS - ESP32 - — ESP-IDF Programming Guide latest documentation (espressif.com) or;

- micropython/ports/esp32 at master · micropython/micropython (github.com)

Micropython File Structures

Download Micropython: https://github.com/micropython/micropython/releases

Steps:

- Unzip micropython file:

tar -xvf  tar -xvf micropython-1.11.tar.gz

- Change directory to your micropython folder “micropython/”

- Compile the micropython cross-compiler folder

make -C mpy-cross

- Edit makefile for our board and port

cd micropython/ports/esp32/

nano Makefile

ESPIDF = /home/(USER)/esp/esp-idf

PORT = /dev/ttyUSB0

- From console add desired dependency (i.e. Camera, ADC) from ESP Registry

idf.py add-dependency "espressif/esp32-camera^2.0.3"

- Compile Makefile

make

Connecting USB through WSL

Reference: You can now connect USB devices in Windows Subsystem for Linux under Windows 11 (xda-developers.com)

- Install the latest usbipd-win (.msi) from https://github.com/dorssel/usbipd-win/releases/latest

- From wsl console install the user space tools for USB/IP and a database of USB hardware identifiers:

sudo apt install linux-tools-generic hwdata

sudo update-alternatives --install /usr/local/bin/usbip usbip /usr/lib/linux-tools/*-generic/usbip 20

- Open windows command prompt and list all attached devices

usbipd wsl list

- Attach ports that we need to WSL

usbipd wsl attach --busid <busid>

- Open WSL console again and list the attached USB in WSL

lsusb

- Erase the existing firmware on ESP32

Make erase

- Deploy to the ESP32 Board

Make deploy

Add Python library to device before build the firmware

Add Camera Driver to Micropython

- Copy files inside “/board” to “micropython/ports/esp32/boards” by cloning file from lemariva/micropython-camera-driver: add camera support to MicroPython (github.com)

- Clone the camera driver (micropython-camera-driver) with the same level of micropython folder (lemariva/micropython-camera-driver: add camera support to MicroPython (github.com))

- Copy esp32-camera components to “~/esp/esp-idf/components”

cd ~/esp/esp-idf/components

git clone https://github.com/espressif/esp32-camera

- Compile the firmware:

cd micropython/ports/esp32

make USER_C_MODULES=../../../../micropython-camera-driver/src/micropython.cmake BOARD=ESP32_CAM all

- Erase the previous firmware inside the device and deploy the new firmware

Make erase

esptool.py --chip esp32 --port /dev/ttyUSB0 write_flash -z 0x1000 build-ESP32_CAM/firmware.bin

- Connect to ESP32-Cam board, then press reset button

picocom -b 115200 /dev/ttyUSB0

- Try to import camera and initialize it

Import camera

Camera.init(0, format=camera.JPEG, fb_location=camera.PSRAM)

Tensorflow Lite in ESP32 (C/C++ Version)

- Clone the tflite-micro-esp-examples (https://github.com/espressif/tflite-micro-esp-examples.git)

- Enter to tflite-micro-esp-examples folder, specifically in examples/person_detection/

Cd  tflite-micro-esp-examples/examples/person_detection

- Setting the camera type to ESP32-CAM by AI-Thinker by entering menuconfig first

Idf.py menuconfig

- Under application configuration -> Camera Configuration -> Select Camera -> ESP32-CAM by AI-Thinker. After that, type S to save the configuration and ESC to escape

- To inference using camera please comment #define CLI_ONLY_INFERENCE 1 inside /examples/person_detection/main/esp_main.h or Uncomment that MACROS to inference on embedded image

- Set the target board and begin to build the firmware

Idf.py set-target esp32

Idf.py clean build

- Give permission to the USB port and flash the image

Sudo chmod 777 /dev/ttyUSB0

Idf.py -p /dev/ttyUSB0 flash

- Inference to detect person using picocom in linux or Thonny from windows

picocom -b 115200 /dev/ttyUSB0

- The model will run automatically to detect person from camera

Tensorflow Lite in ESP32 (Micropython Version)

- Clone repository from https://github.com/mocleiri/tensorflow-micropython-examples.git

- Install python libraries for the virtual environment requirement

Pip3 install wave

Pip3 install pillow

- Enter to tensorflow-micropython-examples folder and download all submodules

Cd tensorflow-micropython-examples

Git submodule init

Git submodule update --recursive

- Regenerate the microlite/tflm directory

Cd tensorflow

../micropython-modules/microlite/prepare-tflm-esp.sh

- Setup micropython libraries

Cd ../micropython

Git init submodule

Git submodule update --recursive

- Compile the cross compiler in micropython

Cd mpy-cross

make

- Clone the camera-driver-examples to replace the old one

Cd micropython-modules

Rm -rf camera-driver-examples

Git clone https://github.com/lemariva/micropython-camera-driver

- Edit micropython-modules/micropython.cmake on camera-driver directory path to resemble like this

- Edit some code in micropython-modules/microlite/tensorflow/tensorflow-microlite.c

Line 215 from

const mp_obj_type_t microlite_tensor_type = {

{ &mp_type_type },

.name = MP_QSTR_tensor,

.print = tensor_print,

.locals_dict = (mp_obj_dict_t*)&tensor_locals_dict,

};

to

MP_DEFINE_CONST_OBJ_TYPE(

microlite_tensor_type,

MP_QSTR_tensor,

MP_TYPE_FLAG_NONE,

print, tensor_print,

locals_dict, (mp_obj_dict_t*)&tensor_locals_dict

);

Line 264 from

const mp_obj_type_t microlite_audio_frontend_type = {

{ &mp_type_type },

.name = MP_QSTR_audio_frontend,

.make_new = af_make_new,

.print = af_print,

.locals_dict = (mp_obj_dict_t*)&audio_frontend_locals_dict,

};

To

MP_DEFINE_CONST_OBJ_TYPE(

microlite_audio_frontend_type,

MP_QSTR_audio_frontend,

MP_TYPE_FLAG_NONE,

make_new, af_make_new,

print, af_print,

locals_dict, (mp_obj_dict_t*)&audio_frontend_locals_dict

);

Line 414 from

const mp_obj_type_t microlite_interpreter_type = {

{ &mp_type_type },

.name = MP_QSTR_interpreter,

.print = interpreter_print,

.make_new = interpreter_make_new,

.locals_dict = (mp_obj_dict_t*)&interpreter_locals_dict,

};

To

MP_DEFINE_CONST_OBJ_TYPE(

microlite_interpreter_type,

MP_QSTR_interpreter,

MP_TYPE_FLAG_NONE,

print, interpreter_print,

make_new, interpreter_make_new,

locals_dict, (mp_obj_dict_t*)&interpreter_locals_dict

);

- Begin to build the firmware. Change directory to tensorflow-micropython examples\boards\esp32\MICROLITE_SPIRAM_CAM

Idf.py set-target esp32

Idf.py clean build

Idf.py flash

- After successfully flashing the firmware, upload these necessary files to the device. In my case, I was using Thonny to upload files. (Ps. These files are from /examples/person_detection)

- Finally, do inference by typing “import person_detection_cam” in the shell. The LED will flash if the model detect person in the camera, morover the confidence threshold in the default setting is 10 and could be adjusted depend on the needs.